Blind Setting

- Steps

  - Register VeraEdge Gateway into the Vera user account.

  - Create RFXtrx controller device in the VeraEdge

  - Create Blind controller device in the VeraEdge

  - Pairing the Blind controller with the Blind motor

  - Update Room Metadata of the SRS

- Register VeraEdge Gateway into the Vera user account.

  - Vera User Account: telkomsel / telkomsel@2018

  - Connect VeraEdge to an internet enabled LAN cable

  - Access http://home.getvera.com and log in

  - Find VeraEdge and register into the account

    - Your Laptop (or Desktop) and the VeraEdge must be in the same network.

      - Add Another Controller

- Select ‘VeraEdge’

- Find Vera Controller by S/N

- Click ‘Next Step’

- Wait for VeraEdge to update

- Go to dashboard when update is complete

- Click ‘Complete Setup’

- Registration is complete.

- Do not upgrade VeraEdge firmware

- Click ‘OK’ when you see below

- Create RFXtrx controller device in the VeraEdge

  - Download REXtrx device definition files from http://code.mios.com/trac/mios_rfxtrx/changeset/153/trunk?old_path=%2F&format=zip

And extract the zip file. It contains 38 files.

  - Connect into the VeraEdge from the http://home.getvera.com

  - On VeraEdge WEB UI: Goto ‘Apps > Develop apps > Luup files’ and upload all files.

  - To create virtual RFXtrx device in VeraEdge, goto ‘Apps > Develop apps > Create device’ and enter ‘D_RFXtrx.xml’ into ‘Upnp Device Filename’ filed and click ‘Create device’ button on the bottom. (Wait about 2 minutes for VeraEdge to reboot)

  - Refresh the WEB page and go to the menu ‘Device’. ‘RFXtrx controller device’ should appear. But if you don’t see it, please restart VeraEdge(power off, on) and repeat creation virtual RFXtrx device

  - Connect RFXtrx device with VeraEdge using serial cable

  - Go to the menu ‘Apps > Develop apps >Serial port configuration’ and set properties like below

    - Path = /dev/ttyUSB0

    - Baud = 38400

    - Used by device = RFXtrx controller device

  - Click ‘Save’ button to apply the settings.

- Create Blind controller device in the VeraEdge

  - To create virtual blind motor controller, go to the menu ‘Device’ and go in to the settings of ‘RFXtrx controller device’

  - On the settings menu list, click the menu ‘New device’ and set the properties like below

    - Category = RFY

    - Device Type = Window covering

    - Device name = {FloorNo}-{RoomName}-ch:{BlindChannelNumber}

ex) 8F-WarRoom-ch:1

    - Id = Enter the room ID. (Refer to Excel File provided)

ex) Room ID, 'War Room' = 213

    - Unit code = Blind channel number (Range = 1~4)

ex) Channel 1 = 1

    - On the menu ‘Device’, you should see the virtual blind controller, like below.

- Paring the Blind Motor with the Blind Controller

  - Prepare the blind remote controller of the room.

  - Make sure the blind is well controlled by the remote control

  - Change the blind motor to the PROGRAM mode.

    - To change the mode, press the PROGRAM button for 2 seconds using a spiky pin. The button is on the back side of the remote controller (At that time the blind motor should jog, briefly up&down).

  - Go to the settings of ‘RFXtrx controller device’, and go in to the ‘Manage device’

  - Select the blind control device to pair with the blind motor in the PROGRAM mode and click ‘Run’ button. (At that time the blind motor should jog, briefly up&down). If jogs, they are paired.

  - Test the blind control by default web UI on the menu ‘Device’

- Update Room Metadata of the SRS

  - Prepare the VeraEdge IP address and virtual blind controller ID

    - Check the virtual blind controller ID from VeraEdge WEB UI

Ex) Go in ‘Devices > 8F-WarRoom-ch:1 > Advanced > Params’ and check the value of the field ‘id’

  - Update the Room information

    - Access http://sary.sofila.eduframe.co.kr:8080 with user ‘admin / admin’

    - Go to the menu ‘Management’ and find the target room.

    - Update date ‘Gateway Static IP’ and ‘Blind controller ID’ of each blind.

- Update VeraEdge Static IP Address (Do this instruction when Static IP address is confirmed)

  - Go to the VeraEdge WEB UI

    - Access http://home.getvera.com

    - Connect the target VeraEdge

    - Go to Settings > Net&Wi-Fi

    - Choice ‘Manually configure’

    - Change the selection ‘DHCP’ to ‘Static IP’ and fill out the fields with correct values.

    - Disable the option ‘Firewall’

    - Click ‘Save and apply’ button at the bottom. (It will take about 2 minutes)